# App 4 Task Table, WCET Evidence, and Hazard Analysis

## Task Table + WCET Evidence

| Task | Priority | Function | Synchronization Primitive | Estimated WCET | WCET Evidence |
| --- | --- | --- | --- | --- | --- |
| Ground ISR | Interrupt | Detect ground uplink button press | Binary Semaphore | <100 μs | ISR debounce + xSemaphoreGiveFromISR() |
| Uplink Task | 12 | Process incoming ground commands | Binary Semaphore | <1 ms | Semaphore receive, logging |
| Telemetry Writer (Attitude) | 8 | Increment shared telemetry sequence | Mutex | <500 μs | Single critical section increment |
| Telemetry Writer (Thermal) | 8 | Increment shared telemetry sequence | Mutex | <500 μs | Single critical section increment |
| Thruster Tasks (4) | 5 | Request/release RCS valve | Counting Semaphore | <1 ms (excluding burn) | Semaphore operations |
| Fault Handler (H) | 15 | Priority inversion demo | nav_lock | <500 μs (excluding wait) | Measured with esp_timer_get_time() |
| Star Tracker (M) | 10 | CPU workload | None | ≈24.55 s | Measured wall-clock |
| Orbit Calculator (L) | 5 | CPU workload holding lock | nav_lock | ≈12.27 s | Measured lock hold time |
| Telemetry Monitor | 3 | Detect lost updates | None | <1 ms | Runs every 5 s |

## Hazard Analysis

| Hazard | Cause | System Effect | Severity | Likelihood | Mitigation |
| --- | --- | --- | --- | --- | --- |
| Lost telemetry updates | Mutex removed | Incorrect packet numbering | High | Medium | Protect with mutex |
| Priority inversion | High task blocked by low task | Missed deadlines | High | Medium | Priority inheritance mutex |
| Thruster starvation | 4 tasks, 3 valves | Delayed maneuver | Medium | Low | Counting semaphore |
| Missed uplink | ISR not processed | Command ignored | High | Low | Binary semaphore |
| Resource contention | Shared hardware | Latency | Medium | Medium | Synchronization primitives |
| Button bounce | Mechanical switch | False commands | Low | Medium | 200 μs debounce |
| Race condition | Unsynchronized shared variable | Corrupt telemetry | High | Medium | Mutex + monitor |
| Deadlock | Lock not released | System stall | High | Low | Single-lock design |
| CPU overload | Long demo workload | Lower-priority delays | Low | Medium | Watchdog/isolated demo |